# ANNEX II + III:	TECHNICAL SPECIFICATIONS + TECHNICAL OFFER

# Contract title:Supply of Personal computers	p 1 /…

# Publication reference:05-1050/2

Columns 1-2 should be completed by the contracting authority

Columns 3-4 should be completed by the tenderer

Column 5 is reserved for the evaluation committee

Annex III - the contractor's technical offer

The tenderers are requested to complete the template on the next pages:

- Column 2 is completed by the contracting authority shows the required specifications (not to be modified by the tenderer),

- Column 3 is to be filled in by the tenderer and must detail what is offered (for example the words ‘compliant’ or ‘yes’ are not sufficient)

- Column 4 allows the tenderer to make comments onits proposed supply and to make eventual references to the documentation

The eventual documentation supplied should clearly indicate (highlight, mark) the models offered and the options included, if any, so that the evaluators can see the exact configuration. Offers that do not permit to identify precisely the models and the specifications may be rejected by the evaluation committee.

The offer must be clear enough to allow the evaluators to make an easy comparison between the requested specifications and the offeredspecifications.

LOT 1: Supply of Equipment for Daily center Unit for the elderly

| 1. Item number | 2. Specificationsrequired | 3. Specificationsoffered | 4. Notes, remarks, ref to documentation | 5. Evaluation committee’s notes |
| --- | --- | --- | --- | --- |
| 1 | Personal computers (8) - Laptops Minimum requirements: - Display: 15.6" (39.6 cm) - Resolution: (1920 x 1080) FHD IPS SlimBezel (or similar) - Processor: Intel® Core™ i7-12650H (or similar) - Graphics: Intel® UHD Graphics (or similar) - RAM Memory: 32GB LPDDR4 (or similar) - Disk (Storage): 512GB PCIe NVMe SSD (or similar) - No operating system - 2x2 AX+BT - HD TNR Camera - Battery: 50Wh Li-ion battery - Charger: 90W |  |  |  |